Function FlotRec(SG As Double, SpPower As Double, SpGasRate As Double, AirFraction As Double, SlurryFraction _

As Double, ParticleZPot As Double, BubbleZPot As Double, NumCells As Integer, RetTime As Double, CellVolume As Double, _

FrothHeight As Double, Frother As Integer, FrotherConc As Double, ParticleDiam As Double, Grade As Double, _

ContactAngle As Double, Permitivity As Double, Dielectric As Double, Pe As Double, WaterOrParticle As String, _

dblCellArea As Double, dblBblRatio As Double) As Double

Dim dblVolBubble As Double, dblVolParticle As Double, dblVolBP As Double

Dim dblMassBubble As Double, dblMassParticle As Double, dblMassBP As Double, dblMassTotal As Double

Dim dblCollisionDiam, dblNumAttached As Double

Dim dblKinVisc As Double                                        'kinematic viscosity

Dim dblBulkZone As Double, dblImpellerZone As Double, dblDetach_F As Double         '2 compartment model (Lu)

Dim dblVolImpZone As Double

Dim dblEMean As Double, dblEBulk As Double, dblEImpeller As Double                  'energy dissipations

Dim dblU1Bulk As Double, dblU2Bulk As Double, dblU1Mean As Double, dblU2Mean As Double

Dim dblBeta As Double, dblNParticle As Double, dblNBubble As Double, dblZBubbParticle As Double

Dim dblWorkAdhesion As Double, dblKineticEAttach As Double, dblKineticEDetach As Double

Dim gammaMIBC As Double, gammaPPG400 As Double, gammaOctanol As Double, gammaPentanol As Double

Dim kMIBC As Double, kPPG400 As Double, kOctanol As Double, kPentanol As Double

Dim dblPAtt As Double, dblPDet As Double, dblPCol As Double, dblRe As Double                   'probabilities

Dim dblParticleDens As Double

ParticleDiam = ParticleDiam * 0.000001  'micron to meters

dblH_c_Factor = 150

dblBulkZone = 0.5

dblImpellerZone = 15

dblDetach_F = 0.5            'adjustable parameter for fitting

dblParticleDens = SG * 1000 'x1000 for kg/m^3

dblVolImpZone = 0.1         'set impeller zone 1/10

SpPower = SpPower * 1000    'x1000 for w/m^3

SpGasRate = SpGasRate / 100 '/100 for m/s

gammaMIBC = 0.000005        'mol/m^2

gammaPPG400 = 0.000001      'mol/m^2

gammaOctanol = 0.000008     'mol/m^2

gammaPentanol = 0.000006    'mol/m^2

kMIBC = 230                 'M^-1

kPPG400 = 1700000           'M^-1

kOctanol = 2200             'M^-1

kPentanol = 55              'M^-1

'===== Froth Parameters =====

Dim dblR_Entrainment As Double, dblR_Attachment As Double, dblFrothRecoveryFactor As Double     'entrainment and attachment recoveries

Dim dblB As Double              'fitting parameter

Dim dblAlpha As Double          'fitting parameter

Dim dblCoverage As Double       'max particle coverage attached in froth

Dim dblPFTransfer As Double, dblP_i As Double, dblPr As Double

Dim dblEiw As Double, dblEka As Double

Dim dblRmax As Double

Dim dblBubbleF As Double

dblCoverage = 0.5

dblBubbleF = 0.5

'=====Surface tension calculations=====

If Frother = 2 Then  'MIBC

FrotherConc = FrotherConc / 102170    'convert ppm to mol/L

dblSurfaceTension = 0.07243 - 8.314 * (273.15 + 23) * gammaMIBC * Log(kMIBC * FrotherConc + 1)

ElseIf Frother = 3 Then 'PPG 400

FrotherConc = FrotherConc / 134170    'convert ppm to mol/L

dblSurfaceTension = 0.07243 - 8.314 * (273.15 + 23) * gammaPPG400 * Log(kPPG400 * FrotherConc + 1)

ElseIf Frother = 4 Then  'Octanol

FrotherConc = FrotherConc / 130230    'convert ppm to mol/L

dblSurfaceTension = 0.07243 - 8.314 * (273.15 + 23) * gammaOctanol * Log(kOctanol * FrotherConc + 1)

ElseIf Frother = 5 Then  'Pentanol

FrotherConc = FrotherConc / 88150     'convert ppm to mol/L

dblSurfaceTension = 0.07243 - 8.314 * (273.15 + 23) * gammaPentanol * Log(kPentanol * FrotherConc + 1)

Else

dblSurfaceTension = 0.07243 'pure water @ 23°C

End If

'===== Energy Dissipation =====

dblTotalDens = AirFraction * airDensity + (1 - AirFraction) * SlurryFraction * dblParticleDens _

+ (1 - SlurryFraction) * waterDensity

dblEMean = SpPower / dblTotalDens

dblEBulk = dblBulkZone * dblEMean

dblEImpeller = dblImpellerZone * dblEMean

dblBubbleDiam = dblBubbleF * (2.11 * dblSurfaceTension / (waterDensity * dblEImpeller ^ 0.66)) ^ 0.6

dblNumAttached = dblCoverage * 4 * (dblBubbleDiam / ParticleDiam) ^ 2  'num of particles attached to one bubble

'===== Cell Calculations =====

dblCollisionDiam = 0.5 * (ParticleDiam + dblBubbleDiam)         'avg diam of collision

dblVolParticle = (4 / 3) * PI * (ParticleDiam / 2) ^ 3      'vol 1 part.

dblVolBubble = (4 / 3) * PI * (dblBubbleDiam / 2) ^ 3       'vol 1 bubb.

dblVolBP = dblVolBubble + dblVolParticle                    'vol of 1 BP aggregate

dblKinVisc = waterViscosity / waterDensity

dblMassParticle = dblParticleDens * dblVolParticle          'mass 1 part.

dblMassBubble = airDensity * dblVolBubble                   'mass 1 bubb.

dblMassBP = dblMassBubble + dblMassParticle                 'mass of 1 BP aggregate

dblMassTotal = CellVolume * dblTotalDens

'===== Velocities by Dissipation =====

dblU1Bulk = (0.4 * (dblEBulk ^ (4 / 9)) * (ParticleDiam ^ (7 / 9)) _

* (dblKinVisc ^ (-1 / 3)) * (dblParticleDens / waterDensity - 1) ^ (2 / 3)) ^ 2 'for attachment

dblU2Bulk = 2 * (dblEBulk * dblBubbleDiam) ^ (2 / 3)

dblU1Mean = (0.4 * (dblEMean ^ (4 / 9)) * (ParticleDiam ^ (7 / 9)) _

* (dblKinVisc ^ (-1 / 3)) * (dblParticleDens / waterDensity - 1) ^ (2 / 3)) ^ 2

dblU2Mean = 2 * (dblEMean * dblBubbleDiam) ^ (2 / 3)

dblBeta = (2 ^ (3 / 2)) * (PI ^ 0.5) * (dblCollisionDiam ^ 2) * _

Sqr(dblU1Bulk + dblU2Bulk)       'from Abrahamson model using bulk dissipation

'===== Calc # Density of Bubbles =====

dblNBubble = AirFraction / dblVolBubble

dblNParticle = (1 - AirFraction) * SlurryFraction / dblVolParticle

dblZBubbParticle = dblBeta * dblNBubble * dblNParticle

dblWorkAdhesion = dblSurfaceTension * PI * (ParticleDiam / 2) ^ 2 _

* (1 - Cos(ContactAngle * (PI / 180))) ^ 2      'calc work of adhesion for 1 particle

'===== Energy Barrier =====

Call EnergyBarrier(ParticleDiam, Dielectric, Permitivity, ContactAngle, BubbleZPot, ParticleZPot)   'calls function to calc energy barrier

If dblEnergyBarrier <= 0 Then

dblEnergyBarrier = 0

End If

'===== Kinetic Energy of Attachment =====

dblKineticEAttach = 0.5 * dblMassParticle * dblU1Bulk / (dblDragBeta ^ 2)

dblKineticEDetach = 0.5 * dblMassParticle * (dblDetach_F * (ParticleDiam _

+ dblBubbleDiam) * Sqr(dblEImpeller / dblKinVisc)) ^ 2

'===== Probabilities =====

dblPAtt = Exp(-dblEnergyBarrier / dblKineticEAttach)    'prob. of attachment

dblPDet = Exp(-(dblWorkAdhesion + dblEnergyBarrier) / dblKineticEDetach) 'prob. of detachment

dblRe = Sqr(dblU2Bulk) * dblBubbleDiam / dblKinVisc    'bubble Reynold's number

dblPCol = WorksheetFunction.Tanh(Sqr(3 / 2 * (1 + (3 / 16 * dblRe) / (1 + 0.249 * dblRe ^ 0.56))) _

* (ParticleDiam / dblBubbleDiam)) ^ 2    'prob. collision, modified Luttrell and Yoon

If dblPCol >= 1 Then

dblPCol = 1

End If

dblEiw = gravity / (4 * PI) * (waterViscosity ^ 3 / dblEBulk) ^ 0.25

dblEka = (dblMassBubble * dblU2Bulk - 2 * (dblBubbleDiam / ParticleDiam) ^ 2 * dblMassParticle * dblU1Bulk) ^ 2 _

/ (100 * (dblMassBubble + 2 * (dblBubbleDiam / ParticleDiam) ^ 2 * dblMassParticle))

dblP_i = 13 * Sqr((9 * waterViscosity ^ 2) / (dblBubbleDiam * dblSurfaceTension * dblTotalDens))

dblPr = Exp(-dblEiw / dblEka)

dblPFTransfer = 1 'dblP_i * (1 - dblPr)

'===== Froth Recovery =====

'''===start new froth rec model

Dim dblCoverageFactor As Double

Dim dblQair As Double, dblQliq As Double

Dim dblTopBubbleDiam As Double

Dim dblFrothRetTime As Double

Dim dblPfr As Double, dblBuoyantDiam As Double

dblB = 2.3

'dblTopBubbleDiam = dblBubbleDiam * 10

dblTopBubbleDiam = dblBubbleDiam * (1 / dblBblRatio)

dblCoverageFactor = 2 * (dblBubbleDiam / (ParticleDiam)) ^ 2

dblBuoyantDiam = dblBubbleDiam * ((1 - 0.001275) / ((SG - 1) * dblCoverageFactor)) ^ (1 / 3)

dblPfr = Exp(dblB * ParticleDiam / dblBuoyantDiam)

dblRmax = dblBubbleDiam / dblTopBubbleDiam

dblFrothRetTime = FrothHeight / SpGasRate * dblPfr

dblAlpha = 0.05   'dblCoverageFactor * 6 * SpGasRate / dblBubbleDiam * (1 - dblBubbleDiam / dblTopBubbleDiam) * (ParticleDiam / dblBubbleDiam) ^ 2

dblR_Attachment = dblRmax * Exp(-dblAlpha * dblFrothRetTime)

dblQair = SpGasRate * dblCellArea

dblQliq = CellVolume / (RetTime / NumCells * 60)

dblR_Water_max = (dblQair / dblQliq) / ((1 / 0.2) - 1)

If dblR_Water_max > 1 Then

dblR_Water_max = 0.1

End If

dblR_Entrainment = dblR_Water_max * Exp(-0.0325 * (dblParticleDens - waterDensity) / 1000 - 0.063 * ParticleDiam * 1000000)

dblFrothRecoveryFactor = dblR_Entrainment + dblR_Attachment

'===== Rate Constant =====

Dim dblRecovery_ci As Double

Dim dblRecovery_I As Double

Dim Aa As Double

dblRateConst = dblBeta * dblNBubble * dblPAtt * dblPCol * (1 - dblPDet) * 60    'x60 to make 1/min

dblRecovery_ci = 1 - (1 + dblRateConst * RetTime) ^ (-1)      'eq 32 (Do & Yoon)

'    Aa = (1 + 4 * dblRateConst * RetTime / (NumCells * Pe)) ^ 0.5

'    dblRecovery_ci = 1 - 4 * Aa * Exp(Pe / 2) / ((1 + Aa) ^ 2 * Exp(Aa * Pe / 2) - (1 - Aa) ^ 2 * Exp(-Aa * Pe / 2))

dblRecovery_I = dblRecovery_ci * dblFrothRecoveryFactor / (dblRecovery_ci * _

dblFrothRecoveryFactor + 1 - dblRecovery_ci)            'eq 6.2 finch & dobby

Select Case WaterOrParticle

Case "Water"

FlotRec = dblR_Water_max

Case "Particle"

FlotRec = 1 - (1 - dblRecovery_I) ^ NumCells

End Select

End Function

Function ColFlotRec(SG As Double, SpPower As Double, SpGasRate As Double, AirFraction As Double, SlurryFraction _

As Double, ParticleZPot As Double, BubbleZPot As Double, NumCells As Integer, RetTime As Double, CellVolume As Double, _

FrothHeight As Double, Frother As Integer, FrotherConc As Double, ParticleDiam As Double, Grade As Double, _

ContactAngle As Double, Permitivity As Double, Dielectric As Double, Pe As Double, WaterOrParticle As String, _

dblCellArea As Double, dblBblRatio As Double) As Double

Dim dblVolBubble As Double, dblVolParticle As Double, dblVolBP As Double

Dim dblMassBubble As Double, dblMassParticle As Double, dblMassBP As Double, dblMassTotal As Double

Dim dblCollisionDiam, dblNumAttached As Double

Dim dblKinVisc As Double                                        'kinematic viscosity

Dim dblBulkZone As Double, dblImpellerZone As Double, dblDetach_F As Double         '2 compartment model (Lu)

Dim dblVolImpZone As Double

Dim dblEMean As Double, dblEBulk As Double, dblEImpeller As Double                  'energy dissipations

Dim dblU1Bulk As Double, dblU2Bulk As Double, dblU1Mean As Double, dblU2Mean As Double

Dim dblBeta As Double, dblNParticle As Double, dblNBubble As Double, dblZBubbParticle As Double

Dim dblWorkAdhesion As Double, dblKineticEAttach As Double, dblKineticEDetach As Double

Dim gammaMIBC As Double, gammaPPG400 As Double, gammaOctanol As Double, gammaPentanol As Double

Dim kMIBC As Double, kPPG400 As Double, kOctanol As Double, kPentanol As Double

Dim dblPAtt As Double, dblPDet As Double, dblPCol As Double, dblRe As Double                   'probabilities

Dim dblParticleDens As Double

ParticleDiam = ParticleDiam * 0.000001  'micron to meters

dblH_c_Factor = 150

dblBulkZone = 0.5

dblImpellerZone = 15

dblDetach_F = 0.5             'adjustable parameter for fitting

dblParticleDens = SG * 1000 'x1000 for kg/m^3

dblVolImpZone = 0.1         'set impeller zone 1/10

SpPower = SpPower * 1000    'x1000 for w/m^3

SpGasRate = SpGasRate / 100 '/100 for m/s

gammaMIBC = 0.000005        'mol/m^2

gammaPPG400 = 0.000001      'mol/m^2

gammaOctanol = 0.000008     'mol/m^2

gammaPentanol = 0.000006    'mol/m^2

kMIBC = 230                 'M^-1

kPPG400 = 1700000           'M^-1

kOctanol = 2200             'M^-1

kPentanol = 55              'M^-1

'===== Froth Parameters =====

Dim dblR_Entrainment As Double, dblR_Attachment As Double, dblFrothRecoveryFactor As Double     'entrainment and attachment recoveries

Dim dblB As Double              'fitting parameter

Dim dblAlpha As Double          'fitting parameter

Dim dblCoverage As Double       'max particle coverage attached in froth

Dim dblPFTransfer As Double, dblP_i As Double, dblPr As Double

Dim dblEiw As Double, dblEka As Double

Dim dblRmax As Double

Dim dblBubbleF As Double

dblCoverage = 0.3

dblBubbleF = 0.5

'=====Surface tension calculations=====

If Frother = 2 Then  'MIBC

FrotherConc = FrotherConc / 102170    'convert ppm to mol/L

dblSurfaceTension = 0.07243 - 8.314 * (273.15 + 23) * gammaMIBC * Log(kMIBC * FrotherConc + 1)

ElseIf Frother = 3 Then 'PPG 400

FrotherConc = FrotherConc / 134170    'convert ppm to mol/L

dblSurfaceTension = 0.07243 - 8.314 * (273.15 + 23) * gammaPPG400 * Log(kPPG400 * FrotherConc + 1)

ElseIf Frother = 4 Then  'Octanol

FrotherConc = FrotherConc / 130230    'convert ppm to mol/L

dblSurfaceTension = 0.07243 - 8.314 * (273.15 + 23) * gammaOctanol * Log(kOctanol * FrotherConc + 1)

ElseIf Frother = 5 Then  'Pentanol

FrotherConc = FrotherConc / 88150     'convert ppm to mol/L

dblSurfaceTension = 0.07243 - 8.314 * (273.15 + 23) * gammaPentanol * Log(kPentanol * FrotherConc + 1)

Else

dblSurfaceTension = 0.07243 'pure water @ 23°C

End If

'===== Energy Dissipation =====

dblTotalDens = AirFraction * airDensity + (1 - AirFraction) * SlurryFraction * dblParticleDens _

+ (1 - SlurryFraction) * waterDensity

dblEMean = SpPower / dblTotalDens

dblEBulk = dblBulkZone * dblEMean

dblEImpeller = dblImpellerZone * dblEMean

dblBubbleDiam = dblBubbleF * (2.11 * dblSurfaceTension / (waterDensity * dblEImpeller ^ 0.66)) ^ 0.6

dblNumAttached = dblCoverage * 4 * (dblBubbleDiam / ParticleDiam) ^ 2  'num of particles attached to one bubble

'===== Cell Calculations =====

dblCollisionDiam = ParticleDiam + dblBubbleDiam             'avg diam of collision

dblVolParticle = (4 / 3) * PI * (ParticleDiam / 2) ^ 3      'vol 1 part.

dblVolBubble = (4 / 3) * PI * (dblBubbleDiam / 2) ^ 3       'vol 1 bubb.

dblVolBP = dblVolBubble + dblVolParticle                    'vol of 1 BP aggregate

dblKinVisc = waterViscosity / waterDensity

dblMassParticle = dblParticleDens * dblVolParticle          'mass 1 part.

dblMassBubble = airDensity * dblVolBubble                   'mass 1 bubb.

dblMassBP = dblMassBubble + dblMassParticle                 'mass of 1 BP aggregate

dblMassTotal = CellVolume * dblTotalDens

'===== Velocities by Dissipation =====

dblU1Bulk = (0.4 * (dblEBulk ^ (4 / 9)) * (ParticleDiam ^ (7 / 9)) _

* (dblKinVisc ^ (-1 / 3)) * (dblParticleDens / waterDensity - 1) ^ (2 / 3)) ^ 2 'for attachment

dblU2Bulk = 2 * (dblEBulk * dblBubbleDiam) ^ (2 / 3)

dblU1Mean = (0.4 * (dblEMean ^ (4 / 9)) * (ParticleDiam ^ (7 / 9)) _

* (dblKinVisc ^ (-1 / 3)) * (dblParticleDens / waterDensity - 1) ^ (2 / 3)) ^ 2

dblU2Mean = 2 * (dblEMean * dblBubbleDiam) ^ (2 / 3)

dblBeta = (2 ^ (3 / 2)) * (PI ^ 0.5) * (dblCollisionDiam ^ 2) * _

Sqr(dblU1Bulk + dblU2Bulk)       'from Abrahamson model using bulk dissipation

'===== Calc # Density of Bubbles =====

dblNBubble = AirFraction / dblVolBubble

dblNParticle = (1 - AirFraction) * SlurryFraction / dblVolParticle

dblZBubbParticle = dblBeta * dblNBubble * dblNParticle

dblWorkAdhesion = dblSurfaceTension * PI * (ParticleDiam / 2) ^ 2 _

* (1 - Cos(ContactAngle * (PI / 180))) ^ 2      'calc work of adhesion for 1 particle

'===== Energy Barrier =====

Call EnergyBarrier(ParticleDiam, Dielectric, Permitivity, ContactAngle, BubbleZPot, ParticleZPot)   'calls function to calc energy barrier

If dblEnergyBarrier <= 0 Then

dblEnergyBarrier = 0

End If

'===== Kinetic Energy of Attachment =====

dblKineticEAttach = 0.5 * dblMassParticle * dblU1Bulk / (dblDragBeta ^ 2)

dblKineticEDetach = 0.5 * dblMassParticle * (dblDetach_F * (ParticleDiam _

+ dblBubbleDiam) * Sqr(dblEImpeller / dblKinVisc)) ^ 2

'===== Probabilities =====

dblPAtt = Exp(-dblEnergyBarrier / dblKineticEAttach)    'prob. of attachment

dblPDet = Exp(-(dblWorkAdhesion + dblEnergyBarrier) / dblKineticEDetach)   'prob. of detachment

dblRe = Sqr(dblU2Bulk) * dblBubbleDiam / dblKinVisc    'bubble Reynold's number

dblPCol = WorksheetFunction.Tanh(Sqr(3 / 2 * (1 + (3 / 16 * dblRe) / (1 + 0.249 * dblRe ^ 0.56))) _

* (ParticleDiam / dblBubbleDiam)) ^ 2    'prob. collision, modified Luttrell and Yoon

If dblPCol >= 1 Then

dblPCol = 1

End If

dblEiw = gravity / (4 * PI) * (waterViscosity ^ 3 / dblEBulk) ^ 0.25

dblEka = (dblMassBubble * dblU2Bulk - 2 * (dblBubbleDiam / ParticleDiam) ^ 2 * dblMassParticle * dblU1Bulk) ^ 2 _

/ (100 * (dblMassBubble + 2 * (dblBubbleDiam / ParticleDiam) ^ 2 * dblMassParticle))

dblP_i = 13 * Sqr((9 * waterViscosity ^ 2) / (dblBubbleDiam * dblSurfaceTension * dblTotalDens))

dblPr = Exp(-dblEiw / dblEka)

dblPFTransfer = 1 'dblP_i * (1 - dblPr)

'===== Froth Recovery =====

'''===start new froth rec model

Dim dblCoverageFactor As Double

Dim dblQair As Double, dblQliq As Double

Dim dblTopBubbleDiam As Double

Dim dblFrothRetTime As Double

Dim dblPfr As Double, dblBuoyantDiam As Double

dblB = 2.2

dblTopBubbleDiam = dblBubbleDiam * dblBblRatio

dblCoverageFactor = 2 * (dblBubbleDiam / (ParticleDiam)) ^ 2

dblBuoyantDiam = dblBubbleDiam * ((1 - 0.001275) / ((SG - 1) * dblCoverageFactor)) ^ (1 / 3)

dblPfr = Exp(dblB * ParticleDiam / dblBuoyantDiam)

dblRmax = dblBubbleDiam / dblTopBubbleDiam

dblFrothRetTime = FrothHeight / SpGasRate * dblPfr

dblAlpha = 0.05   'dblCoverageFactor * 6 * SpGasRate / dblBubbleDiam * (1 - dblBubbleDiam / dblTopBubbleDiam) * (ParticleDiam / dblBubbleDiam) ^ 2

dblR_Attachment = dblRmax * Exp(-dblAlpha * dblFrothRetTime)

dblQair = SpGasRate * dblCellArea

dblQliq = CellVolume / (RetTime / NumCells * 60)

dblR_Water_max = (dblQair / dblQliq) / ((1 / 0.2) - 1)

If dblR_Water_max > 1 Then

dblR_Water_max = 0.15

End If

dblR_Entrainment = dblR_Water_max * Exp(-0.0325 * (dblParticleDens - waterDensity) / 1000 - 0.063 * ParticleDiam * 1000000)

dblFrothRecoveryFactor = dblR_Entrainment + dblR_Attachment

'===== Rate Constant =====

Dim dblRecovery_ci As Double

Dim dblRecovery_I As Double

Dim Aa As Double

dblRateConst = dblBeta * dblNBubble * dblPAtt * dblPCol * (1 - dblPDet) * 60    'x60 to make 1/min

dblRecovery_ci = 1 - (1 + dblRateConst * RetTime) ^ (-1)      'eq 32 (Do & Yoon)

'    Aa = (1 + 4 * dblRateConst * RetTime / (NumCells * Pe)) ^ 0.5

'    dblRecovery_ci = 1 - 4 * Aa * Exp(Pe / 2) / ((1 + Aa) ^ 2 * Exp(Aa * Pe / 2) - (1 - Aa) ^ 2 * Exp(-Aa * Pe / 2))

dblRecovery_I = dblRecovery_ci * dblFrothRecoveryFactor / (dblRecovery_ci * _

dblFrothRecoveryFactor + 1 - dblRecovery_ci)            'eq 6.2 finch & dobby

Select Case WaterOrParticle

Case "Water"

ColFlotRec = dblR_Water_max

Case "Particle"

ColFlotRec = 1 - (1 - dblRecovery_I) ^ NumCells

End Select

End Function

Function CycloneRec(Size As Double, CutSize As Double, Alpha As Double, WaterRec As Double) As Double

'Lynch-Rao Hydrocyclone model, with entrainment due to water recovery

Dim X As Double, Y As Double

X = Size / CutSize

Y = (Exp(Alpha * X) - 1) / (Exp(Alpha * X) + Exp(Alpha) - 2)

CycloneRec = Y * (1 - WaterRec) + WaterRec

End Function

Function GrindLib(TonnageInput As Variant, GradeInput As Variant, LibIntensity As Double) As Variant

'Grinding model based on mass balance and liberation intensity.  Liberation intensity indicates what portion of middling material is liberated.

Dim GrindLibTemp(4) As Double

Dim Tonnage(4) As Double

Dim Grade(4) As Double

Dim i As Integer

For i = 1 To 4

Tonnage(i) = TonnageInput(i)

Grade(i) = GradeInput(i)

Next i

'Material in class = material in + material broken in - material broken out.  Grade in = grade out

GrindLibTemp(1) = Tonnage(1) + (LibIntensity / 100 * Tonnage(2) * Grade(2) - LibIntensity / 100 * Tonnage(2) * Grade(4)) / (Grade(1) - Grade(4)) _

+ (LibIntensity / 100 * Tonnage(3) * Grade(3) - LibIntensity / 100 * Tonnage(3) * Grade(4)) / (Grade(1) - Grade(4))

GrindLibTemp(2) = Tonnage(2) - Tonnage(2) * LibIntensity / 100

GrindLibTemp(3) = Tonnage(3) - Tonnage(3) * LibIntensity / 100

GrindLibTemp(4) = Tonnage(4) + (LibIntensity / 100 * Tonnage(2) * Grade(2) - LibIntensity / 100 * Tonnage(2) * Grade(1)) / (Grade(4) - Grade(1)) _

+ (LibIntensity / 100 * Tonnage(3) * Grade(3) - LibIntensity / 100 * Tonnage(3) * Grade(1)) / (Grade(4) - Grade(1))

GrindLib = GrindLibTemp

End Function

Function GrindBreak(TonnageInput As Variant, TopSizeInput As Variant, BottomSizeInput As Variant, SelectionInput As Variant, BreakIntensity As Double) As Variant

Dim GrindBreakTemp(10, 1) As Variant

Dim Tonnage(10, 1) As Double

Dim TopSize(10, 1) As Double

Dim BottomSize(10, 1) As Double

Dim i As Integer, j As Integer

Dim SizeClass As Integer

Dim Selection(10, 10) As Double

Dim Breakage(10, 10) As Double

Dim identity(10, 10) As Double

Dim ColumnSum As Double

Dim BS As Variant

Dim X(10, 10) As Double

Dim Y(10, 10) As Double

Dim LowCycles As Integer

Dim HighCycles As Integer

SizeClass = 10

ColumnSum = 0

For i = 1 To 10

Tonnage(i, 1) = TonnageInput(i)

TopSize(i, 1) = TopSizeInput(i)

BottomSize(i, 1) = BottomSizeInput(i)

If i > 1 Then

If Tonnage(i, 1) = 0 And Tonnage(i - 1, 1) <> 0 Then

SizeClass = i - 1

End If

End If

Next i

For i = 1 To SizeClass

If i = 1 Then

Breakage(i, 1) = 1 - (1 - Exp(-BottomSize(i, 1) / TopSize(1, 1))) / (1 - Exp(-1))

Else

Breakage(i, 1) = (1 - Exp(-BottomSize(i - 1, 1) / TopSize(1, 1))) / (1 - Exp(-1)) - (1 - Exp(-BottomSize(i, 1) / TopSize(1, 1))) / (1 - Exp(-1))

End If

Next i

For i = 1 To SizeClass

For j = 1 To SizeClass

If i <> 1 And j <> SizeClass And j <> 1 Then

Breakage(j, i) = Breakage(j - 1, i - 1)

End If

If j = SizeClass Then

Breakage(j, i) = 1 - ColumnSum

Else

ColumnSum = ColumnSum + Breakage(j, i)

End If

Selection(i, j) = SelectionInput(i)

If i = j Then

identity(i, j) = 1

Selection(i, j) = SelectionInput(i)

Else

identity(i, j) = 0

Selection(i, j) = 0

End If

Next j

ColumnSum = 0

Next i

BS = Application.WorksheetFunction.MMult(Breakage, Selection)

For i = 1 To SizeClass

For j = 1 To SizeClass

X(j, i) = BS(j, i) + identity(j, i) - Selection(j, i)

Next j

Next i

LowCycles = Application.WorksheetFunction.Floor(BreakIntensity, 1)

HighCycles = LowCycles + 1

Dim ProdLow As Variant

Dim ProdHigh As Variant

Dim temp As Variant

temp = X

If LowCycles = 0 Then

ProdLow = Tonnage

Else

For i = 1 To LowCycles - 1

temp = Application.WorksheetFunction.MMult(temp, X)

Next i

ProdLow = Application.WorksheetFunction.MMult(temp, Tonnage)

End If

temp = X

If HighCycles = 0 Then

ProdHigh = Tonnage

Else

For i = 1 To HighCycles - 1

temp = Application.WorksheetFunction.MMult(temp, X)

Next i

ProdHigh = Application.WorksheetFunction.MMult(temp, Tonnage)

End If

For i = 1 To SizeClass

GrindBreakTemp(i, 1) = ProdHigh(i, 1) * (BreakIntensity - LowCycles) + ProdLow(i, 1) * (HighCycles - BreakIntensity)

Next i

GrindBreak = GrindBreakTemp

End Function

Operating Parameters

Feed Matrix