Process Model — Information Flow

The model optimizes unit operation topology for NPV given a ROM ore description. Each stage is a pure function Stream -> Stream (mass, water, size, Cu). The optimizer searches over which stages are on and their operating points; the TEA module converts the steady-state streams into cashflow.

# 1. Structure: three layers

2a - 2b Stage layer. First-principles units (crusher, SAG/BM, hydrocyclone, flotation, regrind, cleaner, tank leach, heap leach, SX, EW, thickener, filter, neutralization). Each is a pure function with a small parameter dataclass.

2. Flowsheet layer. flowsheet.py wires the stages by mass balance on solids, water, F80, and Cu. Topology gates enforce route validity; flotation XOR heap, leach requires cleaner conc, SX requires a leach path, etc.

3. Decision layer. Differential-evolution optimizer sweeps topology on/off bits + continuous operating variables; objective = NPV; the TEA module turns streams + power into revenue, opex, capex, and IRR/NPV.

# 2. How information flows

# 2a High Level Model Information Flow

| File | Accomplishes | Flows or used by: |
| --- | --- | --- |
| __main__.py | Constructs Optimizer(feed_tph, feed_grade) and calls .run() | optimizer.py |
| optimizer.py | Optimizer.run(topologies=None, …), returns OptimizationResult - list of TopologyResult sorted by NPV | topology.py, throughput.py, and tea.py |
| toplogy.py | all_topologies() enumerates valid combos; _is_valid() enforces mutual exclusivity and archetypes() returns the curated ~12 industrial combos | throughput.py and tea.py |
| throughput.py | Runs stage by stage simulation using helpers defined in 3a. Returns streams, kw per stage, mass balance, | tea.py |
| flowsheet.py | Pure functions including crusher.py, sag.py, screen.py, hydrocyclone.py, flotation.py, thickener.py, filter.py, leach.py, sx.py, ew.py, neutralization.py. | tea.py |
| tea.py | Reads from simulator, takes in topology, power, streams, minerology, byproducts, conc_arsenic_pct (smelter penalty) and computes revenue, capex, opex, and returns NPV. | result returned to Optimizer._objective as the -NPV objective value |

# 2b. Simulation orchestration – throughput.py

| Order | Helper function | Flow |
| --- | --- | --- |
| 1 | # _run_crushing | # flowsheet.run_crusher -> crusher.total_power |
| 2 | # _run_screen_and_sag | # flowsheet.run_sag -> sag.bond_wi / t10 (+ optional screen) |
| 3 | # _run_grinding | # flowsheet.run_hydrocyclone -> hydrocyclone.cut_size_plitt (closed BM-cyclone loop) |
| 4 | # _run_rougher_flotation | # flowsheet.run_flotation -> flotation.recovery |
| 5 | # _run_regrind_and_cleaner | # regrind sizing + second flowsheet.run_flotation pass |
| 6 | _run_dewatering | # flowsheet.run_thickener + flowsheet.run_filter |
| 7 | # _run_concentrate_hydromet (if topo enables) | # leach.size_leach -> flowsheet.run_sx (sx.steady_state_mixer) -> ew.size_ew -> neutralization.size_neutralization |
| 7’ | # _run_heap_leach_route (alt route) | # heap_leach.size_heap -> SX -> EW |
| 8 | # _build_mass_balance | # Cu accounting across streams |

# 2c

| Stage | Inputs | Outputs | Drives in TEA |
| --- | --- | --- | --- |
| ROM ore | tph, Cu grade, mineralogy, byproducts | Feed Stream (F80=300mm) | Ore purchase $/t |
| Crusher | Feed F80, target P80 (150mm) | Coarser Stream, kW | Power, capex |
| SAG + BM | Wi, Ecs split, CL=2.5, target P80 (µm) | Ground Stream, kW | Power, mill capex |
| Hydrocyclone | Plitt geometry, slurry % | UF/OF streams, d50, Rv | Capex (small) |
| Flotation (Ro) | k, τ, Rmax by mineralogy | Conc + tails, R, kW | Cell capex, power |
| Regrind + Cleaner | ΔP80, target g=26% Cu, recycle algebra | Cleaner conc + middlings | Capex, power |
| Tank leach (POX) | k(T), P_O2, t; pox_max ceiling | Cu in PLS, residue | O2, acid, autoclave capex |
| Heap leach | E, acid kg/t, pad geometry | PLS, residue on pad | Pad capex, acid opex |
| SX–EW | Moreno isotherm, Faraday CE, bleed b=3% | Cathode tph, raffinate | Reagents, EW power |
| Dewater / Neut. | Galvez flux, Ruth filter, lime stoich. | Conc cake (9% H2O), gypsum | Lime, capex |
| TEA | All streams, kW, capex, Cu/Mo/Au/Ag prices, TC/RC, As | NPV, IRR, payback | — |

# 2d Flowsheet layer

| Boundary | What Balances | Gate / Recycle Rule |
| --- | --- | --- |
| Crusher -> SAG | solids_tph, water = 0 | Always on |
| BM ↔ Cyclone | CL = 2.5 (UF returns to BM) | cyclone_enabled forces CL > 1 |
| Cyclone OF -> Rougher | F80 = d50_OF, water from slurry % | flotation_enabled (always true unless heap) |
| Rougher conc -> Cleaner | Cu mass conserved; Wills recycle to g = 26 % | cleaner_enabled; n_cleaner_stages compounds gangue rejection |
| Rougher tails | Cu and solids to TSF (or tank leach) | Routed to TSF; or to tank leach if leach_enabled |
| Cleaner conc -> POX | Cu_in = Cu_aq + Cu_residue | leach_enabled requires cleaner_enabled |
| PLS -> SX -> EW | Cu mass: aq -> organic -> cathode | sx_enabled + ew_enabled; bleed b = 3 % |
| EW raffinate -> Neut. | H₂SO₄ + Ca(OH)₂ -> CaSO₄·2H₂O | neutralization_enabled requires sx_enabled |
| Heap pad -> SX | Whole-ore Cu, no flotation in chain | heap_leach_enabled excludes flotation_enabled |

# 2e Decision layer

| Variable | # Bounds | # Source / Role |
| --- | --- | --- |
| Outer loop; topology bit-vector | # Enumerated | # topology.all_topologies() filtered by _is_valid |
| Inner loop; DE, 10-D | # scipy.optimize.differential_evolution | # DECISION_VARS in optimizer.py |
| sp_power | # 0.8 – 1.2 kW/m³ | # Flotation cell specific power (Wills Ch. 12) |
| sp_gas_rate | 0.8 – 1.5 cm/s | Superficial gas velocity |
| frother_conc | 10 – 50 ppm | Frother dose |
| flot_residence_time_min | 15 – 20 minN | Bank-total |
| air_fraction | 0.10 – 0.25 | Cell air hold-up |
| slurry_fraction | 0.15 – 0.30 | Solids volume fraction |
| contact_angle | 20 – 55 | Effective hydrophobicity |
| circulating_load | 1.5 – 4.0 | BM / cyclone CL |
| tailings_adverse_factor | 1.0 – 5.0 | TSF capex scalar |
| target_flot_P80_um | 75 – 200 µm | Grind target into rougher |
| Objective | minimize −NPV @ 8 %, 25-yr annuity | tea.evaluate(...)['npv'] |
| Feasibility | topology gates + 1e12 penalty | topology._is_valid, optimizer._FAILURE_PENALTY |

# 2f Current Optimized Parameters

| Variable | Chosen | Bound | Position |
| --- | --- | --- | --- |
| sp_power | 1.200 | 0.8 – 1.2 kW/m³ | At max |
| sp_gas_rate | 1.500 | 0.8 – 1.5 cm/s | At max |
| frother_conc | 50.0 | 10 – 50 ppm | At max |
| flot_residence_time_min | 17.96 | 15 – 20 minN | Interior |
| air_fraction | .250 | 0.10 – 0.25 | At max |
| slurry_fraction | .150 | 0.15 – 0.30 | At min |
| contact_angle | 55.0 | 20 – 55 | At max |
| circulating_load | 1.500 | 1.5 – 4.0 | At min |
| tailings_adverse_factor | 1.000 | 1.0 – 5.0 | At min |
| target_flot_P80_um | 200.0 | 75 – 200 µm | At max |

# 2g Previous Approach

| Variable | Previously Attempted | Pentalty |
| --- | --- | --- |
| sp_power | N | Turbulent detachment term |
| sp_gas_rate | N | Gas cost |
| frother_conc | N | Dose priced reagent |
| flot_residence_time_min | N/A | N/A |
| air_fraction | N | Froth overload instability |
| slurry_fraction | N | Coupling cell volume + water cost |
| contact_angle | IP | Power, acid use, water use |
| circulating_load | N | Power cost |
| tailings_adverse_factor | N | Recovery |
| target_flot_P80_um | N | Recovery curve |

| Layer | Modules | Owns / Decides | Gets from | Hands to | Contract |
| --- | --- | --- | --- | --- | --- |
| Decision | Optimizer.py, topology.py | Topology bits + 10 operating vars; max NPV | NPV, TEA | Topology, Params, Flowsheet | Topology, Params |
| Flowsheet | Flowsheet.py, throughput.py | Routing, mass balance, validity gates | Topology, params, decision | Stream, stage, TEA | Stream, validity gates |
| Stage | Stage files | Unit physics, pure Stream | Stream, params, flowsheet | Stream, KW, flowsheet | Stream, KW |

# 3. Assumptions

| Area | Choice | Why |
| --- | --- | --- |
| Scope | Concentrator-only, ROM in, conc or cathode out | Mine + G&A folded into ore $/t — keeps model size to one capex envelope |
| Stage shape | Pure functions Stream -> Stream | Composable, easy to enable/disable, no hidden state between stages |
| Cleaner circuit | Wills recycle algebra, target g = 26% Cu | Matches smelter-acceptable concentrate without iterating per cell |
| Hydromet ceilings | POX/heap caps by mineralogy | Prevents chalcopyrite heap winning on bad kinetics |
| Capex | Power-law + Lang 5.0, no separate EPCM/contingency | Lang already bundles installation; transparent single multiplier |
| Cashflow | Pre-tax flat annuity, 25 yr, 8% | Strips tax/financing noise from route selection |
| Optimizer | DE, 10 box-bounded vars, no nonlinear constraints | Robust on a non-smooth NPV surface; topology gates handle feasibility |

# 4. Why this structure

Separation of physics from economics. Stages output streams + kW; TEA maps streams + kW + capex to NPV. Either side can be re-fit (e.g., a new flotation model) without touching the other.

Mass balance as the integrator. The Stream dataclass is the only contract between stages, so adding a new unit (e.g., dense-media separator) only needs a new Stream -> Stream function plus a topology rule.

Topology gates Infeasible flowsheets are pruned before they cost a physics evaluation; the optimizer only sees the feasible NPV surface.

Mineralogy as the single ore key. All ore-specific behavior (flotation max, POX/heap caps, byproducts, As) is keyed off one string so a project override propagates consistently.