# 1. Scope & structure

- Concentrator-only: ROM ore in, Cu concentrate or cathode out. No mine, labor, or G&A; supplier mine cost folded into the ore-purchase coefficient.

- Three product routes: (1) sulfide flotation, (2) concentrate leach, (3) whole-ore heap leach.

- Byproducts credited only on the sulfide route; zero on hydromet/heap.

# 2. Comminution

Bond third theory (crusher, SAG, ball, regrind):

E_cs = 10·Wi·(1/√P_80 − 1/√F_80) kWh/t  kW = E_cs · tph

Inverted for product size: 1/√P_80 = E_cs/(10·Wi) + 1/√F_80 (SME Eq 20.10).

Two-stage grind split: SAG grinds fresh feed at fixed sag_Ecs = 2.8 kWh/t; BM Ecs is the Bond remainder to target P_80 → bm_kw = max (0, Ecs_total − sag_Ecs)·tph. Circulating load sizes BM/cyclone but does not change total grinding power.

- Wi = 14.0 kWh/t (all stages); crusher P_80 = 150 mm; ore F_80 = 300 mm.

- target flotation P_80 = 150 µm; circulating load CL = 2.5; regrind target P_80 = 30 µm.

- SAG/BM JK params: A=70, b=0.7, ta=0.5, PLI=7 (descriptors; no power model in sag.py).

- Crusher (Pothina 2007): idle P0 = V·A_idle·PF; crush P = E·LF/DE·RC; V=4160, PF=0.85, DE=0.95, LF=1.0, A_idle=30.

Regrind mill factors (× ball-mill kWh/t and capex/kW): ball 1.00/1.00; tower 0.75/0.85; isamill 0.60/1.40.

# 3. Classification (hydrocyclone, Plitt 1976 / Samaeili 2017)

d50 = C1 · 50.5·Dc^0.46·Di^0.6·Dx^1.21·e^(0.063·cv) ·√µ / [Du^0.71·h^0.38·Q^0.45·(ρp−ρf)]

S = Qo/Qu (flow split, Eq 13)

Rv = S/(S+1) (water recovery to UF)

m = C4·1.94·e^(−1.58·Rv·(Dx²/Q) ^0.15)

Eff(x) = 1−e^(−0.693·(x/d50) ^m)

- Coeffs: C1=0.0387, C2=51.27, C3=0.1953, C4=0.1762; µ=1.0.

- Geometry (cm): Dc=50, Di=12, Dx=15, Du=8, h=120; P=70 kPa.

- Closed-circuit: all fresh feed reports to overflow at steady state; efficiency evaluated at F_80 as proxy for full distribution; fallback Eff=0.5 if F_80 unknown.

- Cyclone-feed solids 55%; flotation-feed solids 30%.

# 4. Flotation (Mohit Thesis)

Mechanistic rate constant from collision / attachment / detachment:

k = β·n_b·P_att·P_col·(1−P_det) ·60  [1/min]

R_coll = 1 − 1/(1+k·τ) (single CSTR)

R_i = R_coll·R_froth / (R_coll·R_froth + 1 − R_coll)

- Surface tension: σ = σ0 − R·T·Γ·ln(k·c + 1), σ0 = 0.07243 N/m @ 23°C.

- Bubble diameter: d_b = f_bub·[2.11·σ/(ρw·ε_imp^0.66)]^0.6; impeller zone ε_imp = 15·ε_mean.

- Collision kernel β: Attachment P_att = e^(−E_b/E_kin); collision P_col = min (tanh²x, 1); detachment P_det = e^(−(W_adh+E_b)/E_kin).

- Froth recovery: R_froth = min (R_attach + R_entrain, 1).

Structural: rougher bank treated as a single mixing stage with bank-total residence time — R_total = R_i (deliberate departure from VBA 1−(1−R_i)^N; NumCells is sizing-only). Scavenger combine: R_total = R + (1−R) ·R_scav.

- VBA fit: b=2.3, alpha=0.05, bubble_f / detach_f / bulk_zone / coverage = 0.5.

- energy_barrier_j = 1e-18 J — placeholder until full DLVO.

- Rougher recovery cap = 0.90; scavenger-on-tails = 0.50; gangue recovery = 0.10 (calibrated so conc still needs cleaning); conc solids 35%.

- Default frother = MIBC (type 2).

Cleaner circuit (grade-targeting recycle algebra):

R_g,target = R_cu,eff·c_R·(1−g) / [g·(1−c_R)]

R_recycle = R_eff / [1 − R_R·(1−R_eff)]  (Wills 8th ed.)

- liberation factor = clamp(1 − 0.0020·ΔP_80, 0.85, 1.05); per-stage Cu recovery clamped [0.80, 0.99]; per-stage gangue recovery clamped [0.04, 0.20].

- target cleaner conc grade = 26% Cu; cleaner cell 50 m³; residence 12 min; each stage rejects ~50% gangue; Cu assumed full internal recycle (no per-stage Cu loss).

Auto-sizing: total_vol = Q·(t_res/60)·froth_factor; n_cells = ceil(total_vol/cell_vol); cell 500 m³, t_res 18 min, froth_factor 1.2.

Recovery ceilings by mineralogy (flot_R_max):

# 5. Leaching

Tank leach:

k(T) = k0·e^(−Ea/RT)·P_O2^n  dX/dt = 3k·(1−X)^(2/3)  X(t) = 1 − (1−k·t)³

- k0=500, Ea=70 kJ/mol, n=0.7, P_O2=12 bar, T=200°C, 4 h/stage, 1 stage.

- Extraction hard cap MAX_LEACH_EXTRACTION = 0.95.

- Acid: ṁ_acid = cu_leached·1000·6.17 kg/h (H₂SO₄/Cu = 6.17, SME Ch 20).

- Multi-stage approximated as plug flow (total V = stages × per-stage V).

Heap leach (fixed empirical extraction, no kinetics):

cu_leached = ore_tph·g·E  pls_m3/h = (cu_leached·1e6 / pls_gpl)/1000

- E = 0.70 (supergene chalcocite mid-range); acid = 15 kg/t ore (gangue-driven); PLS = 4 g/L Cu; heap 8 m; bulk density 1.7 t/m³; cycle 365 d. Pad area = ore inventory / (ρ·height).

- Not used for chalcopyrite (heap extraction <30%).

# 6. SX–EW

SX mixer + settler:

Y* = A·X* / (X* + B)  A = a·ML  B = (10^−pH)^b / ML^c · (d·[Cu]_PLS + f·[Cu]_BO)

Strip linear: Y* = C·X* + D.

Default pH=1.8, barren-organic Cu=0.5 g/L. Steady state by fixed-point iteration (tol 1e-6).

EW (SME Handbook Ch 20) Faraday cathode mass:

W = I·t·M_Cu·CE / (z·F)  I = i_op·A  i_op = 0.40·i_L  i_L = 18.6·[Cu]_g/L

- Cu in electrolyte 45 g/L; CE = 0.92; cell voltage = I·R + 1.96 V; ρ_elec=0.5 Ω·cm; spacing 5 cm; 1.2 m²/plate; 60 cathodes/cell. Check value ~2.1 kWh/kg Cu.

Steady-state closed recycle (both hydromet branches):

R_ss = R / [1 − (1−R)·(1−b)]  R = η_SX·η_EW = 0.95×0.95  b = 0.03

- 3% electrolyte bleed is the only permanent Cu sink; EW spent loss folded into R_ss.

# 7. Dewatering & neutralization

Thickener:

ψ_M = vTF·C_M·(1−rF·C_M)^n

ψ_max = (4·vTF/rF)·n·(n−1)^(n−1)/(n+1)^(n+1)

A = SF·W/(ρs·ψ_max), SF=1.25

- Tailings thickener unit area 0.30 t/m²/h. Conc UF 65%; tails/thickener default UF 60%.

Belt filter (Perry Ch 18, Ruth Eq 18-71):

θ/(V/A) = µ·α·w/(2·ΔP)·(V/A) + µ·r/ΔP

- Cake moisture is a user-supplied spec (9%), not computed — Perry gives no residual-moisture kinetics. α=1e11, r=1e10, ΔP=70 kPa, min cake 3 mm — placeholder until leaf-test values.

Neutralization (two-stage lime, SME Ch 20 Eq 20.64–20.66): lime/gypsum/precipitate from stoichiometry, mol/h = kg/h·1000/MW. lime purity 0.95; 2 stages/side; 1 h/stage. Fe/As co-precip rate-limited by min(Fe, As); warn if Fe:As < 3:1. Stage-2 base metals lumped at avg MW 175. Residual sulfate load fixed {Cu 100, Zn 50} kg/h.

Screen (SME Ch 20): area = U/(A·B…J); base capacity A=4.5 stph/ft² — placeholder Table 20.8 mid-range. DBD sanity gate ≤ 4× aperture.

- Unknown mineralogy keys default to chalcopyrite.

- Per-project overrides take precedence over tables.

# 8. TEA — capex

Power-law equipment cost:

C = C_basis·(cap/cap_basis)^exp · f_infl

parallel trains when cap_req > cap_max: n = ceil(cap_req/cap_max), C = n·perry_cost(cap_req/n)

Installed:

C_installed = C_equipment·Lang + C_tailings + C_heap  Lang = 5.0

- Tailings & heap pad bypass Lang (already integrated installed).

- M&S inflation default = 2383/1000 = 2.383 (Perry basis → 2025).

- SAG/ball mill: $50M @ 16.5 MW, exp 0.85 (vendor-anchored 2024); single-train caps 28 MW SAG / 22 MW BM (Cobre Panama).

- Flotation cell: Arfania 2017 25351·V^0.452 below 158 m³; large-cell tier $1.5M @ 500 m³, exp 0.50 above.

- Thickener (Parkinson–Mular): $510k @ 10 m, exp 1.6. Belt filter $147.75k @ 10 m², exp 0.58.

- Heap pad $200M @ 16 Mt/yr, exp 0.85. Tailings (O'Hara 1992 Eq 6.3.145): $20,000·T^0.5 (1988 USD)·adverse_factor.

- Screen basis ($150k @ 8 m²) — placeholder.

# 9. TEA — revenue, opex, cashflow

Concentrate (sulfide):

Rev_net = tph_Cu,pay·P_Cu·H − tph_conc·TC·H − tph_Cu,pay·(RC + pen_As)·H

tph_Cu,pay = max(0, tph_Cu,conc − 0.01·tph_conc)·0.965·k_grade

- Grade payable: k_grade = 0 if g<18%; linear ramp to 1 over 18–25%; 1 above 25%.

Cathode (hydromet): Rev = tph_cathode·1.0·P_Cu·H (no TC/RC); concentrate & byproduct revenue zeroed.

Arsenic penalty ($/t Cu): $0 below 0.20% As; linear to $500 at 0.50%; full rejection (penalty = Cu price) at ≥0.50%.

Cashflow:

NPV = −C_total + (Rev_net − Opex)·Σ_{t=1..L} (1+r)^−t  r = 0.08, L = 25 yr

- IRR by bisection over r ∈ [−0.9, 50].

- Prices / rates: Cu $12,500/t; TC $22/t conc; RC $50/t Cu; payable 0.965; Mo $44,000/t (R=0.55); Au $80.4M/t (0.95 pay); Ag $964.5k/t (0.85 pay).

- Opex: power kW·H·$0.05/kWh; water $0.30/m³; reagent $0.50/t ore; grinding media $0.30/kWh; O&M 0.035·capex; hours/yr 7884 (0.9 cap factor).

- Ore purchase (merchant framing): $/t = 10.0 + g_Cu·6000 (Wood Mac 2024 bottom-up; bands $3,800–$6,750/t Cu).

- Hydromet reagent unit costs (acid $150/t, O₂ $100/t, organic $7/L) — placeholders; lime $200/t. O₂ proxy = 1 kg O₂/kg Cu leached — placeholder midpoint.

# 10. Optimizer (differential evolution)

Objective: minimize −NPV at fixed 8% discount; failure penalty 1e12 on exception, non-finite NPV, or overall R < 0.05.

Decision variables (box bounds only, no nonlinear constraints):

| variable | low–high | variable | low–high |
| --- | --- | --- | --- |
| sp_power (kW/m³) | 0.8–1.2 | slurry_fraction | 0.15–0.30 |
| sp_gas_rate (cm/s) | 0.8–1.5 | contact_angle (°) | 20–55 |
| frother_conc (ppm) | 10–50 | circulating_load | 1.5–4.0 |
| flot_residence (min) | 15–20 | tailings_adverse | 1.0–5.0 |
| air_fraction | 0.10–0.25 | target_flot_P80 (µm) | 75–200 |

# 12. Topology validity rules

- flotation XOR heap_leach; leach XOR heap_leach.

- cyclone requires ball_mill; cleaner requires regrind.

- leach requires flotation AND cleaner (autoclaves run on ~25–30% Cu cleaner conc, not 1–3% rougher froth).

- sx requires a leach path; ew requires sx; neutralization requires a leach path.

- heap_leach forces OFF: ball_mill, cyclone, regrind, cleaner, thickener, filter (bypasses fine grind/dewatering).

- regrind_mill_type ∈ {ball, tower, isamill}; n_cleaner_stages ∈ {1,2,3}.

- Heap route suits low-grade oxide / supergene chalcocite at <0.5% Cu.

# 13. Mass balance

cu_in = ore_tph·g

closure = cu_in − Σ cu_out (conc + cathode + tails + raffinate + bleed)

R_overall = (cu_conc + cu_cathode) / cu_in

- If leach on: concentrate Cu = 0; unleached Cu rejoins tails.

- Heap route: residue stays on pad, counted as final tails; emits zero concentrate streams.

One page document that defines information flow through the model

What does the structure look like

What does each input -> output look like

Shows end to end what is the model

How does the information flow look?

What are the assumptions being made?

Why that structure?